Content

2	Classes	1

2.1	Layer1AggregationProfileConfiguration	1

2.2	Layer1AggregationProfileSpec	2

2.3	Layer1AggregationProfileStatus	2

2.4	Layer1AggregationProfile_Pac	3

3	Data Types	4

4	Enumeration Types	4

4.1	ProfileNameType	4

5	Primitive Types	5

# Classes

## Layer1AggregationProfileConfiguration

Applied stereotypes:

- OpenModelClass

- support: MANDATORY

- OpenInterfaceModelClass

- objectCreationNotification: NO

- objectDeletionNotification: NO

Attributes for Layer1AggregationProfileConfiguration

Table 1: Attributes for Layer1AggregationProfileConfiguration

| Attribute Name | Type | Multiplicity | Access | Stereotypes | Description |
| --- | --- | --- | --- | --- | --- |
| aggregationGroupIsOn | Boolean true | 1 | R | OpenInterfaceModelAttribute • AVC: NO • bitLength: NA OpenModelAttribute • partOfObjectKey: 0 • isInvariant: false • valueRange: no range constraint • unit: no unit defined • support: MANDATORY | If true, the layer 1 aggregation group is configured to be activated, so it is forwarding traffic to the serving layers. |
| _clientLtp | LogicalTerminationPoint ./. | 1 | R | PassedByReference OpenInterfaceModelAttribute • AVC: NO • bitLength: NA OpenModelAttribute • partOfObjectKey: 0 • isInvariant: false • valueRange: no range constraint • unit: no unit defined • support: MANDATORY | UUID of the LTP with LayerProtocol::layerProtocolName==LAYER_PROTOCOL_NAME_TYPE_ETHERNET_CONTAINER_LAYER that is configured to be transported by a group of LTPs with LayerProtocol::layerProtocolName==LAYER_PROTOCOL_NAME_TYPE_HYBRID_MW_STRUCTURE_LAYER or LAYER_PROTOCOL_NAME_TYPE_PURE_ETHERNET_STRUCTURE_LAYER. |
| _serverLtpList | LogicalTerminationPoint ./. | 0..* | R | PassedByReference OpenInterfaceModelAttribute • AVC: NO • bitLength: NA OpenModelAttribute • partOfObjectKey: 0 • isInvariant: false • valueRange: no range constraint • unit: no unit defined • support: MANDATORY | List of UUIDs of a group of LTPs with LayerProtocol::layerProtocolName==LAYER_PROTOCOL_NAME_TYPE_HYBRID_MW_STRUCTURE_LAYER or LAYER_PROTOCOL_NAME_TYPE_PURE_ETHERNET_STRUCTURE_LAYER that are configured to transport an LTP with LayerProtocol::layerProtocolName==LAYER_PROTOCOL_NAME_TYPE_ETHERNET_CONTAINER_LAYER. |

## Layer1AggregationProfileSpec

Applied stereotypes:

- OpenModelClass

- support: MANDATORY

- OpenInterfaceModelClass

- objectCreationNotification: YES

- objectDeletionNotification: YES

Attributes for Layer1AggregationProfileSpec

Table 1: Attributes for Layer1AggregationProfileSpec

| Attribute Name | Type | Multiplicity | Access | Stereotypes | Description |
| --- | --- | --- | --- | --- | --- |
| _layer1AggregationProfile_Pac | Layer1AggregationProfile_Pac ./. | 1 | R | OpenInterfaceModelAttribute • AVC: NO • bitLength: NA OpenModelAttribute • partOfObjectKey: 0 • isInvariant: true • valueRange: no range constraint • unit: no unit defined • support: MANDATORY | See referenced class |

## Layer1AggregationProfileStatus

Applied stereotypes:

- OpenModelClass

- support: MANDATORY

- OpenInterfaceModelClass

- objectCreationNotification: NO

- objectDeletionNotification: NO

Attributes for Layer1AggregationProfileStatus

Table 1: Attributes for Layer1AggregationProfileStatus

| Attribute Name | Type | Multiplicity | Access | Stereotypes | Description |
| --- | --- | --- | --- | --- | --- |
| aggregationGroupIsUp | Boolean false | 1 | R | OpenInterfaceModelAttribute • AVC: YES • bitLength: NA OpenModelAttribute • partOfObjectKey: 0 • isInvariant: false • valueRange: no range constraint • unit: no unit defined • support: MANDATORY | If true, the layer 1 aggregation group is currently activated and forwarding traffic to the serving layers. |
| _clientLtp | LogicalTerminationPoint ./. | 1 | R | PassedByReference OpenInterfaceModelAttribute • AVC: YES • bitLength: NA OpenModelAttribute • partOfObjectKey: 0 • isInvariant: false • valueRange: no range constraint • unit: no unit defined • support: MANDATORY | UUID of the LTP with LayerProtocol::layerProtocolName==LAYER_PROTOCOL_NAME_TYPE_ETHERNET_CONTAINER_LAYER that is transported by a group of LTPs with LayerProtocol::layerProtocolName==LAYER_PROTOCOL_NAME_TYPE_HYBRID_MW_STRUCTURE_LAYER or LAYER_PROTOCOL_NAME_TYPE_PURE_ETHERNET_STRUCTURE_LAYER. |
| _serverLtpList | LogicalTerminationPoint ./. | 0..* | R | PassedByReference OpenInterfaceModelAttribute • AVC: YES • bitLength: NA OpenModelAttribute • partOfObjectKey: 0 • isInvariant: false • valueRange: no range constraint • unit: no unit defined • support: MANDATORY | List of UUIDs of a group of LTPs with LayerProtocol::layerProtocolName==LAYER_PROTOCOL_NAME_TYPE_HYBRID_MW_STRUCTURE_LAYER or LAYER_PROTOCOL_NAME_TYPE_PURE_ETHERNET_STRUCTURE_LAYER that are transporting an LTP with LayerProtocol::layerProtocolName==LAYER_PROTOCOL_NAME_TYPE_ETHERNET_CONTAINER_LAYER. |

## Layer1AggregationProfile_Pac

Applied stereotypes:

- OpenModelClass

- support: MANDATORY

- OpenInterfaceModelClass

- objectCreationNotification: NO

- objectDeletionNotification: NO

Attributes for Layer1AggregationProfile_Pac

Table 1: Attributes for Layer1AggregationProfile_Pac

| Attribute Name | Type | Multiplicity | Access | Stereotypes | Description |
| --- | --- | --- | --- | --- | --- |
| _layer1AggregationProfileConfiguration | Layer1AggregationProfileConfiguration ./. | 1 | R | OpenInterfaceModelAttribute • AVC: NO • bitLength: NA OpenModelAttribute • partOfObjectKey: 0 • isInvariant: true • valueRange: no range constraint • unit: no unit defined • support: MANDATORY | See referenced class |
| _layer1AggregationProfileStatus | Layer1AggregationProfileStatus ./. | 1 | R | OpenInterfaceModelAttribute • AVC: NO • bitLength: NA OpenModelAttribute • partOfObjectKey: 0 • isInvariant: true • valueRange: no range constraint • unit: no unit defined • support: MANDATORY | See referenced class |

# Data Types

# Enumeration Types

## ProfileNameType

A controlled list of Profile names.

Contains Enumeration Literals:

- PROFILE_NAME_TYPE_LAYER_1_AGGREGATION_PROFILE: